Name: Shriya Ramya Mohan                                              Exp No: 3

Roll no: 241901106

STUDY THE DIFFERENT TYPES OF NETWORK CABLES AND CRIMPING OF CABLES WITH RJ45 CONNECTOR

AIM:

To study the different types of network cables and crimping of cables with RJ45 connector.

THEORY

RJ45 CONNECTOR:

RJ45 is an 8-pin connector used at the end of Ethernet UTP cables for networking.
It has 8 positions and 8 contacts (8P8C) that carry data signals.
RJ45 connectors follow colour-coding standards—T568A and T568B—for arranging the 8 wires of the UTP cable in a specific order for proper communication.

PROCEDURE:

- Cut the UTP cable to required length.

- Strip 1.5–2 cm of outer jacket using the stripper section of the crimping tool.

- Untwist the 4 pairs and straighten the 8 wires.

- Arrange the wires according to the required standard:

  - T568B for Straight-Through (both ends)

  - T568A on one end + T568B on other end for Cross-Over

- Hold the wires flat and trim them evenly with the cutter part of the crimping tool.

- Insert the wires into RJ45 connector (clip facing down).
Ensure:

  - Each wire sits in its correct slot

  - The colours remain in correct order

  - Wires reach the front tip of connector

- Place connector inside crimping slot of the tool.

- Press the crimping tool firmly to lock the wires into the connector.

- Repeat for the other end of the cable.

- Test the cable using a LAN tester or by connecting two devices.

RESULT

Cables were successfully prepared using UTP cable, RJ45 connectors, and a crimping tool, and their continuity was verified.